3.2 Engineering Models and Simulation

The simulation models the Primary Design Scenario, S3 — Emergency Vehicle Priority, using a closed-loop control architecture:

Sensing → Central Optimisation → Signal Actuation

with emergency-priority and fault-detection/fallback overrides. The model was used to assess whether the proposed system can satisfy the defined system requirements and to compare adaptive control against a conventional fixed-time baseline.

Engineering model

A simplified traffic-flow and queueing model was implemented using a time-varying Poisson arrival process. Vehicles discharge during a green phase at a saturation flow of 1900 veh/h/lane (0.528 veh/s). The simulation uses a 1 s timestep over a 30-minute peak period. Opposing approaches operate together as two signal phases: N+S and E+W.

The fixed-time baseline uses Webster's signal-timing method, with a timing plan calculated from the average demand and then kept constant. The adaptive controller instead uses vehicle-actuated logic: a phase must satisfy the minimum green time, can extend while vehicles remain queued, terminates when the queue clears, and is limited by the maximum green time. This allows the controller to respond to changing traffic demand rather than following a fixed cycle.
The sensing and communication loop is represented using bounded stochastic delays. Data updates occur every 3–5 s, with detection and transmission delays centred around approximately 1 s. These values are modelling assumptions used to test the system requirements rather than measured hardware performance.

Emergency and fault handling

Emergency vehicle priority follows the precedence rule safety > compliance > emergency > efficiency. If a pedestrian clearance interval is active, an emergency request is deferred until the clearance completes rather than interrupting the pedestrian crossing. The request is then granted, and the system returns to normal control after the emergency vehicle passes. This ensures that pedestrian clearance is not violated.

The fault model injects a sensor or communications failure and measures the time required to detect the fault, activate fallback control and notify an operator. The fallback mechanism maintains traffic control using the safe fixed-time strategy rather than allowing a single sensor failure to disable the intersection.

Simulation results

The live run demonstrated the complete closed-loop sequence, including sensing, phase changes, pedestrian clearance, emergency priority, fault detection, fallback activation and operator notification.

The 30-minute statistical simulation produced the following results:

| Requirement | Target | Mean | Worst | Result |
| --- | --- | --- | --- | --- |
| SR-F01 Data update | ≤5 s | 4.01 s | 4.99 s | PASS |
| SR-F02 Detection | ≤2 s | 0.99 s | 2.47 s | FAIL |
| SR-F03 Transmission | ≤2 s | 0.90 s | 1.98 s | PASS |
| SR-F04 Emergency requests granted | 100% | 100% | 100% | PASS |
| SR-F05 Priority status | ≤2 s | 0.87 s | 1.48 s | PASS |
| SR-N01 Pedestrian violations | 0 | 0 | 0 | PASS |
| SR-F06 Fault detection | ≤10 s | 3.10 s | 8.20 s | PASS |
| SR-F07 Fallback activation | ≤10 s | 3.95 s | 5.23 s | PASS |
| SR-F06 Operator notification | ≤30 s | 13.55 s | 20.16 s | PASS |
| SR-N04 Availability | ≥99% | 99.80% | 99.80% | PASS |

The adaptive controller produced an average queue of 13.85 vehicles, compared with 17.06 vehicles for the fixed-time baseline. This represents an 18.8% reduction in average queue length.

The main limitation identified was SR-F02. Although the mean detection latency was below the 2 s requirement, the worst observed latency was 2.47 s. This indicates that the detection component has limited margin and would require further optimisation or a more appropriate statistical acceptance criterion before deployment.

Overall, the simulation provides numerical evidence that the proposed closed-loop architecture can respond to changing traffic conditions, handle emergency-priority conflicts safely, and maintain operation following a sensor/communications fault. The model remains a requirements-level engineering simulation rather than a full traffic microsimulation; it does not model individual vehicle positions, car-following or lane-changing behaviour.